# T1-1  Descriptive Analysis — Dataset Overview

This section provides the dataset description for the group report (300–400 words). Every figure/table in the final report must have a caption — placeholders are included below where plots from Sprint 1 should be inserted.

The ASHRAE Great Energy Predictor III (GEPIII) dataset was released as part of a Kaggle machine learning competition hosted by ASHRAE in late 2019. It is the largest open-access building energy meter dataset published to date, comprising over 20 million hourly energy meter readings collected from 2,380 meters across 1,448 buildings situated in 16 sites worldwide. The dataset spans three consecutive years of data (2016–2018), with the first year (2016) provided as the training set and the remaining years withheld as test data during the competition.

The dataset is distributed across five files: train.csv (energy meter readings for 2016), building_metadata.csv, weather_train.csv, test.csv (meter readings for 2017–2018, labels withheld), and weather_test.csv. The target variable is meter_reading — the hourly energy consumption of a building's meter in kilowatt-hours (kWh), with values spanning several orders of magnitude from near-zero readings to tens of thousands of kWh. Due to this wide range, the standard evaluation metric adopted by the competition and all subsequent studies is the Root Mean Squared Log Error (RMSLE), which penalises proportional errors equally across all value scales.

Four distinct meter types are present in the dataset: electricity (meter type 0, 59.7% of all readings), chilled water (20.7%), steam (13.4%), and hot water (6.3%). Buildings span a wide range of primary use types, with Education (41.4%) and Office (22.3%) being the most common. Building floor areas range from a few thousand to over a million square feet, and year of construction spans from the early 1900s to 2018.

The weather data provides hourly readings per site, including air temperature, dew point temperature, sea level pressure, wind direction, wind speed, and cloud coverage. Missing values are common in the weather data (particularly cloud coverage and wind direction) and were addressed during preprocessing using forward-fill and site-level median imputation. Anomalous meter readings — including long streaks of zero values and implausibly large spikes — were identified and removed during the preprocessing step, consistent with the approach taken by the top-ranked competition teams (Miller et al., 2020).

Summary statistics are presented in Table 1 and Figure 1 below. The target variable meter_reading exhibits a right-skewed distribution with a long tail; applying a log1p transformation (log(x+1)) normalises this distribution and is standard practice for this dataset.

## Table 1: Dataset Summary Statistics

| File | Rows | Columns | Date Range | Key Variables |
| --- | --- | --- | --- | --- |
| train.csv | 20,216,100 | 4 | 2016-01-01 to 2016-12-31 | building_id, meter, timestamp, meter_reading |
| building_metadata.csv | 1,449 | 6 | N/A (static) | site_id, primary_use, square_feet, year_built, floor_count |
| weather_train.csv | 139,773 | 9 | 2016-01-01 to 2016-12-31 | air_temperature, dew_temperature, wind_speed, cloud_coverage, sea_level_pressure |
| test.csv | 41,697,600 | 3 | 2017-01-01 to 2018-12-31 | building_id, meter, timestamp |

Table 1: Summary of the five ASHRAE GEPIII data files, including row counts, column counts, date ranges, and key variables.

## [ INSERT FIGURE 1: Meter Type Distribution Bar Chart — from Sprint 1 ]

Figure 1: Distribution of meter readings by meter type. Electricity meters account for the majority (59.7%) of readings in the dataset, followed by chilled water (20.7%), steam (13.4%), and hot water (6.3%).

## [ INSERT FIGURE 2: Log1p Distribution of meter_reading — from Sprint 1 ]

Figure 2: Distribution of the log1p-transformed meter_reading target variable. Applying log(x+1) normalises the heavily right-skewed raw distribution and is the standard transformation used across all top GEPIII solutions.